Hydro-Spatial Modeling of Nature-Based Solutions for Scalable Flood Risk Reduction:
Evidence from Climate-Vulnerable Indian Watersheds

Raj Amritam*, Nidhi Rawat

Department of Environmental Science, DBS Global University, Dehradun 248001, India
*Corresponding Author: raj.amritam@dbsgu.ac.in

Science of the Total Environment, 912, 169234 (2025)

______________________________________________________________________

# Abstract

Scaling Nature-Based Solutions (NbS) for flood management requires robust cost-benefit evidence and transferable methodologies - both critically lacking for the Indian Himalayan Region (IHR). This study quantifies NbS flood mitigation performance against grey infrastructure under current and projected climate conditions in the Alaknanda-Mandakini watershed (12,511 km²), Uttarakhand. Using integrated HEC-HMS and SWAT modeling with CORDEX-SA climate projections (RCP 4.5/8.5), we demonstrate that optimally placed NbS (47 sites, 231 km²) achieves 22-34% peak discharge reduction versus 14-20% for equivalent grey infrastructure investment. Economic analysis reveals NbS benefit-cost ratio of 4.2:1 (INR 85.3 Cr NPV) versus 1.8:1 for grey (INR 28.6 Cr NPV) over 50-year horizon. NbS annual co-benefits total INR 5.8 Crore (carbon sequestration, biodiversity, water quality, livelihoods). Under RCP 4.5 (2050), NbS retains 73% efficacy; under RCP 8.5, 62%. Community surveys (n=450) show 71% NbS support, rising to 82% for hybrid approaches. Framework transferability validated on Bhagirathi basin (AUC=0.87 without re-calibration, 60% less data). Results provide compelling evidence for NbS integration into national flood management policy and demonstrate a scalable methodology for evidence-based planning across the IHR.

Keywords: Nature-Based Solutions; Flood risk reduction; Cost-benefit analysis; Climate change; Hydrological modeling; Scalability; Indian Himalayan Region; Grey infrastructure comparison

______________________________________________________________________

# 1. Introduction

The intensification of extreme hydrological events under climate change demands a fundamental rethinking of flood management paradigms (IPCC AR6, 2022). In India, where floods account for 42% of disaster deaths and INR 25,000 Crore annual damages, the conventional reliance on grey infrastructure - embankments, dams, and channelization - has proven insufficient, particularly in the geologically fragile Indian Himalayan Region (IHR) where engineered structures face accelerated degradation, high maintenance costs, and design capacity exceedance.

Nature-Based Solutions (NbS) address these limitations through ecosystem services that attenuate flood flows, reduce erosion, and provide adaptive capacity. Meta-analyses demonstrate 10-40% peak flow reduction (Chausson et al., 2020), superior long-term economics (Narayan et al., 2016), and significant co-benefits (Seddon et al., 2020). However, three critical barriers impede NbS scaling in India: (1) insufficient quantitative evidence under monsoon-extreme conditions, (2) absence of cost-benefit frameworks incorporating Indian economic conditions and co-benefits, and (3) lack of transferable methodologies for rapid deployment across diverse watersheds.

This paper addresses all three barriers through scenario-based hydrological modeling, comprehensive economic analysis, and transferability validation, providing the evidence base for NbS integration into India's national flood management framework.

# 2. Materials and Methods

2.1 Study Area and NbS Design

The Alaknanda-Mandakini watershed (12,511 km²) in Garhwal Himalaya was selected for its documented flood vulnerability, data availability, and diversity of NbS-suitable terrain. 47 NbS sites were identified through AI-GIS multi-criteria analysis (Amritam & Rawat, 2025a): riparian buffer restoration (12 sites, 18.4 km²), wetland rehabilitation (8 sites, 14.2 km²), slope afforestation (15 sites, 156.7 km²), check dam cascades (9 sites), and terrace enhancement (3 sites, 42.1 km²).

2.2 Hydrological Model Configuration

HEC-HMS 4.11 (event-based, SCS-CN/UH method) calibrated at 4 stations (Cal NSE=0.81, Val NSE=0.77). SWAT2012 (continuous daily, 87 sub-basins) calibrated via SUFI-2 (Cal NSE=0.71 daily/0.82 monthly). NbS interventions parameterized as: CN reduction (afforestation: -15 to -25), Manning's n increase (riparian: 0.03->0.15), storage addition (wetlands: volume×depth), and controlled outflow structures (check dams).

2.3 Climate Scenarios

CORDEX-SA ensemble (5 GCMs × 2 RCPs) bias-corrected via BCSD. Scenarios: Baseline (current), NbS-current, NbS-RCP4.5-2050, NbS-RCP8.5-2050, Grey-equivalent. Return periods: 10, 25, 50, 100 years. Flood extent modeled using HEC-RAS 2D (10m grid).

2.4 Economic Analysis

Cost-benefit framework with 50-year horizon, 8% social discount rate. Costs: capital investment (construction/implementation), annual maintenance, periodic rehabilitation. Benefits: avoided flood damage (damage-probability integration), co-benefits (carbon at INR 1,500/tCO2e from India Carbon Market pricing, biodiversity using habitat equivalency, water quality using replacement cost, livelihoods from household income surveys). Sensitivity analysis: discount rate (5-12%), climate scenario, NbS performance degradation.

2.5 Community Assessment

Household survey (n=450, stratified random across 24 villages in 3 altitude zones), 8 FGDs (gender-segregated), 15 KIIs. Willingness-to-pay estimated via contingent valuation method.

# 3. Results

3.1 Flood Discharge Reduction

NbS reduces peak discharge at Rudraprayag by 31.7% (10-yr RP), 29.0% (25-yr), 25.1% (50-yr), and 22.5% (100-yr). Grey infrastructure achieves 20% up to design capacity, declining to 14% at 100-yr RP due to overtopping. Flood extent (100-yr, HEC-RAS 2D): baseline 48.2 km², NbS 28.4 km² (-41.1%), grey 35.6 km² (-26.1%). Time to peak delayed by 2.5-4.0 hours with NbS (enhanced storage effect).

3.2 Climate Scenario Performance

Under RCP 4.5 (2050): precipitation increases 8-12%, peak discharge increases 15-22% from baseline. NbS maintains 73% of current efficacy (effective reduction: 16-23%). Under RCP 8.5: precipitation increases 15-25%, NbS retains 62% efficacy (14-20% reduction). Critical finding: even under worst-case RCP 8.5, NbS performance matches grey infrastructure's current performance, while grey infrastructure itself degrades further under increased flows.

3.3 Economic Analysis

NbS: Capital INR 24.5 Cr, maintenance INR 1.2 Cr/yr, avoided damage INR 18.4 Cr/yr, co-benefits INR 5.8 Cr/yr. NPV = INR 85.3 Cr, BCR = 4.2:1, IRR = 34%.
Grey: Capital INR 38.2 Cr, maintenance INR 3.8 Cr/yr, avoided damage INR 12.7 Cr/yr. NPV = INR 28.6 Cr, BCR = 1.8:1, IRR = 14%.

Sensitivity: BCR remains >2.5 for NbS across all scenarios (discount rate 5-12%, performance degradation up to 30%). Grey BCR falls below 1.0 at 12% discount rate.

3.4 Community Acceptance

71.3% support NbS, 82.2% support hybrid (NbS + selective grey). Willingness-to-pay: mean INR 1,240/household/year for NbS maintenance. 89% believe forests reduce floods (TEK alignment). Key barriers: land tenure uncertainty (42%), temporal mismatch of benefits (35%), institutional trust deficit (77%).

3.5 Scalability and Transferability

Bhagirathi basin transfer test: AUC = 0.87 (vs 0.946 in training), 74% NbS site overlap with local experts, 60% reduction in required calibration data. Estimated unit cost of framework application decreases 40-60% for subsequent watersheds due to regional parameterization.

# 4. Discussion

4.1 NbS Efficacy in Extreme Conditions

Our finding of 22-34% peak reduction under monsoon extremes exceeds conservative expectations and positions NbS as viable for high-intensity rainfall environments. The mechanism is two-fold: (1) restored vegetation intercepts 15-25% of rainfall before reaching soil surface, and (2) enhanced infiltration in restored soils converts 30-45% of excess surface runoff to subsurface pathways with delayed response. The steep terrain paradoxically amplifies afforestation benefits because vertical intercepted storage is proportionally larger relative to thin soil columns on slopes.

4.2 Economic Case for NbS

The 4.2:1 BCR is among the highest reported globally for NbS flood mitigation, attributable to: (1) high flood damage intensity in IHR (INR 800-2,000 Cr per major event), (2) low NbS implementation costs due to community labor availability, (3) significant co-benefits from biodiversity-rich mountain ecosystems, and (4) long NbS functional life (50-100+ years). This finding provides powerful justification for redirecting a portion of India's INR 25,000 Cr annual flood management budget toward NbS.

4.3 Scalability Implications

The 60% data reduction for transfer application means the HSMF can be rapidly deployed across the IHR's 2,500+ watersheds at fraction of traditional assessment costs. The regional parameterization approach - training on detailed pilot areas and transferring to similar geographies - offers a practical pathway for India's National Mission on NbS.

# 5. Conclusions

This study provides the first comprehensive evidence of NbS flood mitigation performance, economics, and scalability for the Indian Himalayan Region. Key conclusions:

1. NbS achieves 22-34% flood reduction at 36% lower cost than grey infrastructure (BCR 4.2:1 vs 1.8:1), making it economically superior for Himalayan flood management.

2. Co-benefits (INR 5.8 Cr/yr) transform the economics from 'viable' to 'compelling', justifying integration into India's climate adaptation finance.

3. NbS maintains adequate performance (62-73%) under 2050 climate scenarios, demonstrating climate-resilient adaptation value.

4. The methodology is scalable across the IHR at 40-60% reduced unit cost per additional watershed, enabling rapid evidence-based NbS planning at national scale.

5. Community acceptance is high (71-82%) and aligns with traditional ecological knowledge, supporting implementability.

These findings provide quantitative justification for policy integration of NbS into India's flood management framework, with implications for similar mountain regions globally.

# References

Chausson, A., et al., 2020. Glob. Change Biol., 26, 6134-6155.

Dadson, S.J., et al., 2017. Proc. R. Soc. A, 473, 20160706.

IPCC, 2022. AR6 WGII: Impacts, Adaptation and Vulnerability.

IUCN, 2016. Nature-Based Solutions to Address Global Societal Challenges.

Narayan, S., et al., 2016. Proc. Natl. Acad. Sci., 113, 4712-4717.

Seddon, N., et al., 2020. Phil. Trans. R. Soc. B, 375, 20190120.

UNDRR, 2023. Global Assessment Report on Disaster Risk Reduction.